/*

* 3pi-linefollower-pid - demo code for the Pololu 3pi Robot

*

* This code will follow a black line on a white background, using a

* PID-based algorithm.

*

* http://www.pololu.com/docs/0J21

* http://www.pololu.com

* http://forum.pololu.com

*

*/

// The 3pi include file must be at the beginning of any program that

// uses the Pololu AVR library and 3pi.

#include <pololu/3pi.h>

// This include file allows data to be stored in program space.  The

// ATmega168 has 16k of program space compared to 1k of RAM, so large

// pieces of static data should be stored in program space.

#include <avr/pgmspace.h>

// Introductory messages.  The "PROGMEM" identifier causes the data to

// go into program space.

const char welcome_line1[] PROGMEM = " Pololu";

const char welcome_line2[] PROGMEM = "3\xf7 Robot";

const char demo_name_line1[] PROGMEM = "PID Line";

const char demo_name_line2[] PROGMEM = "follower";

// A couple of simple tunes, stored in program space.

const char welcome[] PROGMEM = ">g32>>c32";

const char go[] PROGMEM = "L16 cdegreg4";

// Data for generating the characters used in load_custom_characters

// and display_readings.  By reading levels[] starting at various

// offsets, we can generate all of the 7 extra characters needed for a

// bargraph.  This is also stored in program space.

const char levels[] PROGMEM = {

0b00000,

0b00000,

0b00000,

0b00000,

0b00000,

0b00000,

0b00000,

0b11111,

0b11111,

0b11111,

0b11111,

0b11111,

0b11111,

0b11111

};

// This function loads custom characters into the LCD.  Up to 8

// characters can be loaded; we use them for 7 levels of a bar graph.

void load_custom_characters()

{

lcd_load_custom_character(levels+0,0); // no offset, e.g. one bar

lcd_load_custom_character(levels+1,1); // two bars

lcd_load_custom_character(levels+2,2); // etc...

lcd_load_custom_character(levels+3,3);

lcd_load_custom_character(levels+4,4);

lcd_load_custom_character(levels+5,5);

lcd_load_custom_character(levels+6,6);

clear(); // the LCD must be cleared for the characters to take effect

}

// This function displays the sensor readings using a bar graph.

void display_readings(const unsigned int *calibrated_values)

{

unsigned char i;

for(i=0;i<5;i++) {

// Initialize the array of characters that we will use for the

// graph.  Using the space, an extra copy of the one-bar

// character, and character 255 (a full black box), we get 10

// characters in the array.

const char display_characters[10] = {' ',0,0,1,2,3,4,5,6,255};

// The variable c will have values from 0 to 9, since

// calibrated values are in the range of 0 to 1000, and

// 1000/101 is 9 with integer math.

char c = display_characters[calibrated_values[i]/101];

// Display the bar graph character.

print_character(c);

}

}

// Initializes the 3pi, displays a welcome message, calibrates, and

// plays the initial music.

void initialize()

{

unsigned int counter; // used as a simple timer

unsigned int sensors[5]; // an array to hold sensor values

// This must be called at the beginning of 3pi code, to set up the

// sensors.  We use a value of 2000 for the timeout, which

// corresponds to 2000*0.4 us = 0.8 ms on our 20 MHz processor.

pololu_3pi_init(2000);

load_custom_characters(); // load the custom characters

// Play welcome music and display a message

print_from_program_space(welcome_line1);

lcd_goto_xy(0,1);

print_from_program_space(welcome_line2);

play_from_program_space(welcome);

delay_ms(1000);

clear();

print_from_program_space(demo_name_line1);

lcd_goto_xy(0,1);

print_from_program_space(demo_name_line2);

delay_ms(1000);

// Display battery voltage and wait for button press

while(!button_is_pressed(BUTTON_B))

{

int bat = read_battery_millivolts();

clear();

print_long(bat);

print("mV");

lcd_goto_xy(0,1);

print("Press B");

delay_ms(100);

}

// Always wait for the button to be released so that 3pi doesn't

// start moving until your hand is away from it.

wait_for_button_release(BUTTON_B);

delay_ms(1000);

// Auto-calibration: turn right and left while calibrating the

// sensors.

for(counter=0;counter<80;counter++)

{

if(counter < 20 || counter >= 60)

set_motors(40,-40);

else

set_motors(-40,40);

// This function records a set of sensor readings and keeps

// track of the minimum and maximum values encountered.  The

// IR_EMITTERS_ON argument means that the IR LEDs will be

// turned on during the reading, which is usually what you

// want.

calibrate_line_sensors(IR_EMITTERS_ON);

// Since our counter runs to 80, the total delay will be

// 80*20 = 1600 ms.

delay_ms(20);

}

set_motors(0,0);

// Display calibrated values as a bar graph.

while(!button_is_pressed(BUTTON_B))

{

// Read the sensor values and get the position measurement.

unsigned int position = read_line(sensors,IR_EMITTERS_ON);

// Display the position measurement, which will go from 0

// (when the leftmost sensor is over the line) to 4000 (when

// the rightmost sensor is over the line) on the 3pi, along

// with a bar graph of the sensor readings.  This allows you

// to make sure the robot is ready to go.

clear();

print_long(position);

lcd_goto_xy(0,1);

display_readings(sensors);

delay_ms(100);

}

wait_for_button_release(BUTTON_B);

clear();

print("Go!");

// Play music and wait for it to finish before we start driving.

play_from_program_space(go);

while(is_playing());

}

// This is the main function, where the code starts.  All C programs

// must have a main() function defined somewhere.

int main()

{

unsigned int sensors[5]; // an array to hold sensor values

unsigned int last_proportional=0;

long integral=0;

// set up the 3pi

initialize();

// This is the "main loop" - it will run forever.

while(1)

{

// Get the position of the line.  Note that we *must* provide

// the "sensors" argument to read_line() here, even though we

// are not interested in the individual sensor readings.

unsigned int position = read_line(sensors,IR_EMITTERS_ON);

// The "proportional" term should be 0 when we are on the line.

int proportional = ((int)position) - 2000;

// Compute the derivative (change) and integral (sum) of the

// position.

int derivative = proportional - last_proportional;

integral += proportional;

// Remember the last position.

last_proportional = proportional;

// Compute the difference between the two motor power settings,

// m1 - m2.  If this is a positive number the robot will turn

// to the right.  If it is a negative number, the robot will

// turn to the left, and the magnitude of the number determines

// the sharpness of the turn. proportion was 1/20. Changed 2/25/10 JS

int power_difference = proportional/2000 + integral/10500 + derivative * 1.3 ;

// Compute the actual motor settings.  We never set either motor

// to a negative value.

const int max = 200;

if(power_difference > max)

power_difference = max;

if(power_difference < -max)

power_difference = -max;

if(power_difference < 0)

set_motors(max+power_difference, max);

else

set_motors(max, max-power_difference);

}

// This part of the code is never reached.  A robot should

// never reach the end of its program, or unpredictable behavior

// will result as random code starts getting executed.  If you

// really want to stop all actions at some point, set your motors

// to 0,0 and run the following command to loop forever:

//

// while(1);

}

// Local Variables: **

// mode: C **

// c-basic-offset: 4 **

// tab-width: 4 **

// indent-tabs-mode: t **

// end: **